Club Sandwich Block Diagram and Block Specifications

http://www.io.com/~n5fc/club_sandwich_block_spec.doc

Module: Receiver Band Pass Filter

Function:

Provides filtering at input of receiver to reduce image and reduce unwanted out-of-band signals

Input/Output:

- RF IN: 50 ohms, RCA Jack

- RF OUT: 50 ohms, RCA Jack

Key Specs or Limitations:

Frequency range: To be determined

Misc Notes:

Typically, this is a double-tuned circuit, or a high-Q preselector with a "peaking" control
Adding a switchable attenuator might be useful here also

Module: Receiver Preamp

Function:

Improve noise figure and sensitivity, and provide 10-12 dB of pre-mixer gain

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- RF IN: 50 ohms, RCA Jack

- RF OUT: 50 ohms, RCA Jack

Operator Controls:

- IN/OUT Switch (Optional: to bypass preamp when gain not required)

Key Specs or Limitations:

Gain: 10 - 13 db (fixed)
Noise Figure: < 3 dB

Misc Notes:

May be narrow band (i.e., single band) or broadband 3-30 MHz (for broadband, please equalize gain)

Module: Receiver Mixer

Function:

Convert pre-filtered RF to IF (or to audio if direct-conversion receiver)

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- RF IN: 50 ohms, RCA Jack

- VFO IN: 200 ohms, RCA Jack

- IF OUT: 200 ohms, RCA Jack

Operator Controls:

- No operator controls anticipated

Key Specs or Limitations:

- Frequencies In/Out: To be determined

- Gain: 10-15 dB (fixed, RF to IF Voltage Gain)

- Noise Figure: < 6 db

- Tank Circuits: min 1 tank circuit in output stage (or LPF in D-C rcvr)

- VFO Input: expect 0.25V pk-pk

Misc Notes:

If an active mixer is used (like the NE602/SE612), an output amplifier will probably not be required; if a passive (diode ring) mixer is used, a VFO input amplifier and an IF/AF output amplifier will be required. Amplifiers, if used, should have tank circuits to minimize unwanted responses

Module: Post-Mixer IF Amplifier

Function:

Reduce Noise and provide gain/AGC prior to IF filtering

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- IF IN: 200 ohms, RCA Jack

- IF OUT: 200 ohms, RCA Jack

- AGC IN: Zin=1K; 0 to 2.5V (0V = Max gain, 2.5V = 20dB down)

Operator Controls:

- Manual Gain (Optional, in case AGC not available)

Key Specs or Limitations:

- Frequency: To be determined (Audio if D-C receiver)

- Gain: 6- 10 dB at max gain (i.e., at AGC= 0V)

Misc Notes:

Module: IF Filter

Function:

provide narrow-band filtering at IF Frequency (not required for direct-conversion receiver)

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack (if required)

- IF IN: 200 ohms, RCA Jack

- IF OUT: 200 ohms, RCA Jack

Operator Controls:

- No operator controls anticipated

Key Specs or Limitations:

- Crystal Ladder Filter or equivalent

- Minimum 3 crystals (more preferred)

- Use crystals specified for 20 pf load capacitance

Misc Notes:

Module: Post-Filter IF Amplifier

Function:

Provide gain/AGC after IF filtering

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- IF IN: 200 ohms, RCA Jack

- IF OUT: 200 ohms, RCA Jack

- AGC IN: Zin=1K; 0 to 2.5V (0V = Max gain, 2.5V = 20dB down), RCA Jack

- SAMP OUT(Optional): Zout < 200 ohms, RCA Jack (provides sample of IF for external AGC generation, if used)

Operator Controls:

- Manual Gain (Optional, in case AGC not available)

Key Specs or Limitations:

- Frequency: To be determined (Audio if D-C receiver)

- Gain: 10- 12 dB at max gain (i.e., at AGC= 0V)

Misc Notes:

Module: Product Detector / BFO

Function:

Provide product detection for CW / SSB; includes BFO

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- IF IN: 200 ohms, RCA Jack

- AF OUT: Zo=200 ohms max, RCA Jack

Operator Controls:

- BFO Frequency Adjust (may be a screwdriver trim)

Key Specs or Limitations:

- Frequency: To be determined

- IF-to-Audio Voltage Gain: 10- 15 db (fixed)

- Use crystals specified for 20 pf load capacitance

Misc Notes:

- This stage not used in direct-conversion receiver

- If an active mixer is used (like the NE602/SE612), an output amplifier will probably not be required; if a passive (diode ring) mixer is used, an AF output amplifier will be required. An amplifier, if used, should have low-pass roll-off to minimize hiss and eliminate IF feedthrough to the output.

Module: AF Preamp

Function:

Provide gain for Direct Conversion Receiver (not needed for receiver with product detector)

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- AF IN: Zin >= 2K, RCA Jack

- AF OUT: Zout < 200 ohms, RCA Jack

Operator Controls:

- Gain Control Optional (may be screwdriver trimmer)

Key Specs or Limitations:

Voltage Gain: 10-15 db (fixed)
Output: AC-coupled

Misc Notes:

This stage not required for receiver with product detector stage

Module: Receiver Mute and AF AGC Circuit

Function:

Provide for muting of receiver during keying; optionally provides AGC for receivers with IF AGC (and preferrrably for direct conversion receivers)

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- AF IN: Zin = 10K, Zsource expect 200 ohms, RCA Jack

- AF OUT: Zout <= 200 ohms, Zload = 2K, RCA Jack

- KEY IN: Zin >=10K, driven by CMOS; RCA Jack

- AGC IN (Optional): Zin >= 10K, 0-2.5V, where 0V = max throughput, 2.5V = max attenuation

Operator Controls:

- No operator controls anticipated

Key Specs or Limitations:

- Voltage Gain: -3 to 0db (with AGC = 0V)
Voltage Gain: -20 dB (i.e., minimum loss, more loss is prefrrable) (with AGC = 2.5V)

- Output AC coupled

Misc Notes:

Module: AF Filter

Function:

provide audio bandpass shaping or filtering, as appropriate for CW or SSB

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- AF IN: Zin = 2K, RCA Jack

- AF OUT: Zout < 200 ohms, RCA Jack

- SAMP OUT: Zout < 200 ohms, RCA Jack (2nd buffered output for external AGC generator)

Operator Controls:

- CW/SSB switch

- others as required

Key Specs or Limitations:

- For SSB: LPF with Fc=2.5-3KHz or BPF = 300-3000 Hz
For SSB, midband (1 KHz) gain = 0 dB (i.e., x1)

- For CW: BPF with Fo = 750-800 (fixed, or include that freq if filter is variable)
For CW, Bandwidth (3 dB) 400-500 Hz (fixed)
For CW, Gain at Fo = 4-6 db

- Output : AC- coupled

Misc Notes:

Module: AF Power Amplifier

Function:

Monoraul Audio Amplifier, with two inputs, one for audio, one for sidetone, capable of driving 8-ohm headphones

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- AF IN #1: Zin >= 5K, RCA Jack

- AF IN #2 (Sidetone): Zin >= 5K, RCA Jack

- HEADPHONES: 1/8 " Stereo Jack

- SPKR (Optional): RCA Jack

Operator Controls:

- AF GAIN (pot)

- SIDETONE LEVEL SET (can be screwdriver trimmer)

Key Specs or Limitations:

- 0.5 mV, 1 KHz Sine at input gives full volume audio with AF GAIN control set midway

Misc Notes:

Module: AGC Generator

Function:

Based on an IF or AGC sample provided, generate AGC Voltage to be utilized by AGC-controlled stages

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- SIG IN: Zin >= 20K, RCA Jack

- AGC OUT: Zout <= 200 ohms; RCA Jack

Operator Controls:

- Switch: ON/OFF/MANUAL

Key Specs or Limitations:

- Input signals of 3 V p-p cause maximum AGC output (i.e. 2.5V); no signal causes near 0 V output

- Full Wave Rectified Detection

- Using switch, AGC may be turned ON or OFF, or a Manual Gain pot may be placed in circuit, with output 0-2.5 V

Misc Notes:

Module: Transmit Mixer and Oscillator

Function:

converts VFO frequency to output RF frequency; includes crystal-controlled conversion oscillator

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- VFO IN: Zin = 200 ohms, expect 250 mV p-p RCA Jack

- RF OUT: Zo = 200 ohms, RCA Jack

- KEY IN: Zin >= 10K, RCA Jack (0V = key-down)

Operator Controls:

- No operator controls anticipated

Key Specs or Limitations:

- Gain (VFO to Output Voltage Gain): 10 - 15 db (fixed)

- XTAL controlled conversion Oscillator must be trimmable +/-1 KHz min

- Use crystals specified for 18-20 pf load capacitance

- Keying applies to either the VFO or XTAL oscillator, disabling that oscillator during receive periods

- Key-down enables output

- Backwave < 2 mV p-p

- Minimum 1 tuned tank in output circuit

Misc Notes:

Module: Transmit Driver

Function:

Gain and Filter; provides drive to RF Power Amplifier

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- RF IN: Zin = 200 ohms, expect 0.8 V p-p, RCA Jack

- RF OUT: Zout = 50 ohms, BNC Jack

- KEY IN: Zin >=10K, driven by CMOS, RCA Jack, 0 V = key-down (i.e., transmit)

Operator Controls:

- Drive Level (may be screwdriver trimmer)

Key Specs or Limitations:

- Frequency: To be determined

- All stages should be linear (i.e., class A or AB)

- Expect Input signal of 0.8 V p-p into 200 ohms

- Output Power (at Vsupply=13VDC): 350 mW (10.8 V p-p into 50 ohms)

- At least one stage should have a tuned tank circuit

- Output stage should withstand loads of 5-500 ohms without damage

- Backwave (during key-up) should be at least 25 db down (i.e., < 1 mW)

Misc Notes:

Pay attention to keying waveshape, as this will influence final transmitted signal

Module: RF Power Amplifier

Function:

provide 3-5 Watts RF output

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- RF IN: Zin = 50 ohms, expect 10.8 V p-p, BNC Jack

- RF OUT: Zout = 50 ohms, BNC Jack

- KEY IN: Zin >=10K, driven by CMOS, RCA Jack 0 V = key-down (i.e., transmit)

Operator Controls:

- No operator controls anticipated

Key Specs or Limitations:

- Class AB preferred for future SSB use, Class C OK

- Gain 12-14 with .25-.35 W drive (typ)

- Broadband topology preferred, single band OK

- Push-Pull preferred for 2nd harmonic rejection, single ended OK

- Output stage should withstand loads of 5-500 ohms without damage

- Negligible backwave when un-keyed (< 1 mW)

- Heatsink may extend outside normal form factor

Misc Notes:

Module: Output Low Pass Filter

Function:

Provides harmonic rejection

Input/Output:

- RF IN: 50 ohms, BNC Jack

- RF OUT: 50 ohms, BNC Jack

Operator Controls:

- No operator controls anticipated

Key Specs or Limitations:

- 5-7 pole filter (7 preferred)

- capable of handling 25 watts (to accommodate mismatches without arcing)

Misc Notes:

Module: VFO

Function:

Provides injection for Receiver Mixer or Transmit Mixer, as appropriate; includes R.I.T. offset

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- OUT #1: Zo = 200 ohms, RCA Jack

- OUT #2: Zo = 200 ohms, RCA Jack

- KEY IN: Zin >= 10K, driven by CMOS, RCA Jack
(0V = key-down; i.e., transmit)

Operator Controls:

- Main Tuning

Key Specs or Limitations:

- Frequency: To be determined

- Two independently buffered outputs provide 0.25V p-p (0.30V p-p max) into 200 ohms

- Outputs AC Coupled

- Receiver (output 1) offset automatically engaged during receive

- R.I.T control +/- 2KHz min

- Oscillator and tuning stages powered by linear regulator

- For VFO, min 2-stage buffering; For VXO, min 1-stage buffering

Misc Notes:

Module: Keying Control Circuit

Function:

Provides transmit/receive control signals with sequenced delays, to be used by other circuits that need to be keyed

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- KEY IN: 1/8" Mono Jack
Max Input loading (to keyer): 10K (pull-up to Vsupply)
INput states: open = RX; Ground = TX

- KEY0: CMOS/equiv, RCA Jack; Low on key-down, High on key-up (raw keying)

- KEY1: CMOS/equiv, RCA Jack; Low 1 mSec after key-down, High on key-up (keys TX)

- KEY2: CMOS/equiv, RCA Jack; Low 0.5 mSec after key-down, High 0.5 mSec after key-up (keys VFO)

- KEY3: CMOS/equiv, RCA Jack; Low on key-down, High 1-50 mSec after key-up (RX muting)

Key Specs or Limitations:

Misc Notes:

Trimmer or programmable control of KEY3 delay, to accommodate unknown receiver recovery time

Module: Antenna T/R Switch

Function:

Provides switching for antenna circuirt, between transmit and receive

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack (if power required)

- ANTENNA: 50 ohms (nom); BNC Jack

- RCVR OUT: 50 ohms (nom); RCA Jack

- XMIT IN: 50 ohms; BNC Jack

- TR CONTROL: Control from Keying Control Circuit, RCA Jack
Max Zin = 10K/0.01uF; 
Input Signal States: Open or Vsupply for RX; Ground for TX
NOTE: If switching is RF-sensed, this input may not be needed

Key Specs or Limitations:

- Max TX power 10 Watts

Misc Notes:

Module: Sidetone Generator

Function:

generate keyed CW sidetone for injection into audio chain

Input/Output:

- POWER: 9-15 VDC, 5.5 x 2.5mm power jack

- KEY IN: Zin >=10K, to be driven by CMOS (LO=key-down), RCA Jack

- AF OUT: 1 Volt pk-pk

Operator Controls:

- No operator controls anticipated

Key Specs or Limitations:

- Frequency: 750-800 Hz

- Output Amplitude 1 V p-p (nom)

- Output AC-coupled

- No output when un-keyed

- Sine or near-sine preferred

Misc Notes:

This stage not required in receiver that self-generates sidetone from transmit mixer